Michael Vorburger

Informatique, 2ème

EPFL – Juin 1997

Cours de Programmation IV (Prof. Rapin)

Projet Lisp: Algo. de Newton (Racines d'un Polynôme)

µ1 Explications & Notes	2

1.1 Structure Globale	2

1.2 Améliration non Demandées	3

1.2.1 Division par Zero: Utilisation des NaN dans Fonction newton	3

2 Testes Effectués	4

2.1 Une Fonction Avec Racines Réeles	4

2.2 Une Fonction avec Racines Complexes	5

2.3 Quelques fonctions spéciales	5

3 Codes Source	6

Enoncé de l'Exercice

"Le but du projet est du réaliser une fonction capable de résoudre complètement une équation algébrique (polynômes) à coefficients réels ou complexes avec une erreur relative fixée."

# 1	Explications & Notes

## 1.1	Structure Globale

La solution décompose l'algorithme en le plus de sous-fonctions simples possible:

EQUATION_ALGEBRIQUE est la fonction principale qui efface d'abord des éventuels zéros à la fin de la liste des coefficients en appelant POLYADJUST, cherche ensuite une première approximation initiale à l'aide de FIND-INITIAL-VALUE, trouve la première racine avec la fonction POLYZERO, annule des éventuelles minuscules parties imaginaire dans CORRECT, réduit le polynôme à l'aide de POLYREDUCE et continue de trouver d'autres racines jusqu'à ce que le polynôme réduit est vide.

Les fonctions d'aide, qui sont appelés par une des fonctions énoncé ci-dessus, sont: POLYEVAL, HORNER, NEWTON, SOLVE-2ND-DEGREE, REVREST.

Autres remarques générale:

·		Les fonctions ne font à priori pas de différence entre des polynômes avec coefficients réels ou complexes. Lorsqu'il apparaît une solution comportant une minuscule partie imaginaire, celle-ci est annulé dans CORRECT.

·	 	Aucune *VARIABLE-GLOBAL* et ni setq ni setf ont été utilisé.

·		Des fonctions récursifs ont été préféré aux boucles (dotimes, dolist).

Plus de détails sont à trouver dans le code source en annexe ou tous les fonctions sont bien documentés, parfois même chaque ligne d'une fonction.

Le projet a été développé sur PC (Win NT) avec l'excellente version Lisp Allegro CL 3.0.2 de Franz. Inc. (http://www.franz.com)

## 1.2	Amélioration non Demandées

Il peut arriver que la dérivé dans un point est zéros. L'algorithme de Newton tout simple génère une division par zéros dans ce cas quand il calcule la prochaine itération. La solution présenté en annexe reconnaît ce cas et ajoute un tout petit nombre et réessaie l'itération.

### 1.2.1	Division par Zéro: Utilisation des NaN dans Fonction newton

La fonction NEWTON ne teste pas simplement si la dérivée est nul pour alors rendre NIL:

(if (not (zerop yd))              ; ATTENTION: (null 0.0) ==> NIL!

(- xn (/ y yd) )     ; x[n+1] := x[n] + f(x[n]) / f'(x[n])

)

Pour comprendre ce choix, il est important de réaliser que la division par zéro de deux float (ou doubles) ne génère pas une erreur en Lisp (comme c'est le cas pour la division par zéro des integers) mais résulte simplement en un NaN:

> (/ 1 0)

;; Error: Integer division by zero in #<function 0 #xFE82D8>

> (/ 1.0 0.0)

#.+INFINITY

La fonction POLYZERO ne doit alors pas éviter à tout prix d'utiliser le résultat de NEWTON (par exemple pour le calcul de l'erreur maxabserr) mais peut simplement faire un teste avec nanp pour corriger le valeur de la prochaine itération:

; if x1 could not be calc, re-do with slight correction:

(cond ((nanp x1)

(polyzero polycoeffs (+ x0 epsi) epsi) )

...

# 2	Testes Effectués

La solution a été testé intensivement:

## 2.1	Une Fonction Avec Racines Réels

(embedded OLE graphic left out to save space)

Image 1:	La fonction f(x) = x^3 + x^2 - x -1.

> (equation_algebrique '(-1d0 -1d0 1d0 1d0) 1d-16)

(-0.99999999895571 1.0 -1.00000000104429)

Sans la fonction correct pour annuler les minuscule partie imaginaire, on obtiendra:

> (equation_algebrique '(-1d0 -1d0 1d0 1d0) 1d-8)

(#C(-0.999999991074124 1.68855246158923E-16) #C(1.0 -1.50718097857472E-24) #C(-1.00000000892588 -1.68855244651742E-16))

## 2.2	Une Fonction avec Racines Complexes

(embedded OLE graphic left out to save space)

Image 2:	La fonction f(x) = x^3 + x^2 - x -2. Remarquer que f'(-1)=0

> (equation_algebrique '(-2d0 -1d0 1d0 1d0) 1d-16)

(#C(-1.1027847152003 0.665456951152813) 1.20556943040059 #C(-1.1027847152003 -0.665456951152813))

L'évaluation dans un point ou la dérivée est zéro ne pose pas de problème:

> (polyzero '(-2d0 -1d0 1d0 1d0) -1d0 1d-8)

1.20556943040059

## 2.3	Quelques fonctions spéciales

Les erreurs possibles sont reconnues. S'il n'y pas de solution, les fonctions retournent NIL:

> (equation_algebrique '() 1d-8)

NIL

> (equation_algebrique '(0.0d0) 1d-8)          ; y=0

NIL

> (equation_algebrique '(0.0d0 0.0d0 0.0d0) 1d-8)

NIL

> (equation_algebrique '(3.0d0 0.0d0) 1d-8)	    ; y=3

NIL

> (equation_algebrique '(0.0d0 7.0d0) 1d-8)    ; y=7x

(0)

# 3	Codes Source

newton.lisp

; ==============================================================================

; PROJET LISP

;

; Trouver les racines d'un polynom donnee a l'aide de l'algo. de Newton.

;

; Michael Vorburger, 16.7.97 - 18.7.97

; pour le cours de Programmation IV, Prof. Rapin, EPFL

; ==============================================================================

; =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

; NOTE: A "user" should call only function EQUATION_ALGEBRIQUE because only this

;       guarantees correct "error" checks in parameters. So DON'T be surprised if

;       you call for example function POLYZERO or SOLVE-2ND-DEGREE directly with

;       "non-standard" coefficients and get unexptexted behaviour.

; -=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

; ------------------------------------------------------------------------------

; (equation_algebrique polycoeffs epsi)

;

; This is the main function to find all solutions of a polynomial function.

; Call it with a list of coefficients (ordered by growing powers) as first,

; and the max. rel. error as second argument and you get a list of all solutions.

(defun equation_algebrique (polycoeffs epsi)

(let* (( adjusted-poly  (polyadjust polycoeffs) )

( first-x0       (find-initial-value adjusted-poly) )

)

(if (null adjusted-poly)

;then

nil

;else

(let* ((x       (correct (polyzero adjusted-poly first-x0 epsi)) )

(reduced_poly  (polyreduce adjusted-poly x))

)

(if (not (null reduced_poly))

(cons x (equation_algebrique reduced_poly epsi) )

x )

)

)

)

)

; ------------------------------------------------------------------------------

; (polyadjust polycoeffs)

;

; Removes zeros at the end of the list polycoeffs. This is crucial, because eg.

; '(1 2 0) would be interpreted as polynominal of second degree (1+2x+0x^2)

; instead as the linear (1+2x)

(defun polyadjust (polycoeffs)

(if (not (null polycoeffs))

(if (not (zerop (first (last polycoeffs))))   ; ATTENTION: last returns a

polycoeffs                                ;   LIST (with one element)

(polyadjust (revrest polycoeffs) )        ;        che stupido...

)

)

)

; (revrest list) returns all but the last element of a list.

;

(defun revrest (a-list)

(reverse (rest (reverse a-list))) )

; ------------------------------------------------------------------------------

; (find-initial-value polycoeffs)

;

; Determines an initial value. If the polynomial is at least of 2nd degree,

; use the smaller (seen as absolute values) of the two solution. If this

; is not complex, add a small imaginary part to allow convergence.

;

; If it is a linear or constant function, simply return 0 because function

; polyzero will take special care of all those special cases.

(defun find-initial-value (polycoeffs)

(if (> (length polycoeffs) 2)

(let* ((solution

(solve-2nd-degree polycoeffs) )

(smaller-solution

(if (< (abs (first solution)) (abs (second solution)))

(first solution)

(second solution) ) )

)

(if (realp smaller-solution)

(+ smaller-solution #C(0 1.0d-8) )

smaller-solution

)

)

0.0d0                      ; linear or constant function, return 0

)

)

; ------------------------------------------------------------------------------

; (correct number)

;

; If number is a complex, and if it's imag. part is < 1e-15, discard it.

(defun correct (z)

(if (complexp z)

(if (< (abs (imagpart z)) 1.0e-15)

(realpart z)

z

)

z

)

)

; ------------------------------------------------------------------------------

; (solve-2nd-degree polycoeffs)

;

; This function interpretes the elements of the list polycoeffs as coefficients

; of a quadratic function (ordered by growing powers) and returns a LIST

; containing the two solutions. The might be identical, however. -- Does NOT

; use the standard formula, but one which more appropriate for numeric reasons.

;

; SPECIAL BEHAVIOUR: If the third element of polycoeffs (second degree) is zero,

;                    a "small number" is taken instead. This is only for

;                    find-initial-value and does not play any role when called

;                    from polyzero, because such "pseudo-quadratic" polynomials

;                    would have been adjusted to linear polynomials by

;                    polyadjust.

;

; CROSSREFERENCE NOTE: This function is called from find-initial-value

;                      and polyzero.

(defun solve-2nd-degree (polycoeffs)

(let* ((c  (first polycoeffs)  )

(b  (second polycoeffs) )

(a- (third polycoeffs)  )

(a  (if (zerop a-)  1.0d-8  a-) )

(dr (sqrt (- (* b b) (* 4 (* a c)))) )    ; dr = sqrt(b*b - 4*a*c)

(e  (* 2 c ) )

)

(list (/ e (- (- b) dr) )                  ; (e / (-b) - dr)

(/ e (+ (- b) dr) )   )              ; (e / (-b) + dr)

)

)

; ------------------------------------------------------------------------------

; (polyzero polycoeffs x0 epsi)

;

; Intepretes the list polycoeffs as coefficients (ordered by growing

; powers) of a polynominal function and finds a solution for it; one only.

; This is done by applying Newton's algorithm with x0 as initial value and

; epsi as error which is allowed.

; Returns a number or NIL if no solution could be found.

(defun polyzero (polycoeffs x0 epsi)

(cond ((= (first polycoeffs) 0)   0)                 ; polynomial with const 0

((= (length polycoeffs) 1)  NIL )              ; constant polynomial

((= (length polycoeffs) 2)                     ; linear polynomial

(- (/ (first polycoeffs) (second polycoeffs))) )      ; x=-b/a

((= (length polycoeffs) 3)                     ; quadratic polynomial

(first (solve-2nd-degree polycoeffs)) )

(t (let* (( x1        (newton polycoeffs x0) ) ; next iteration x1

( dx        (abs (- x0 x1)) )        ; diff between x0 and x1

( maxabserr (abs (* x0 epsi)) )      ; maximal absolute error

)

(cond ((nanp x1)                      ; if x1 cannot be calc,

(polyzero polycoeffs (+ x0 epsi) epsi) ) ; correction

((< dx maxabserr)                            ; if precise

x1 )                                    ; then done

(t

(polyzero polycoeffs x1 epsi) )         ; else next

)

)

)

)

)

; (newton polycoeffs xn) calculates the next iteration for Newton's

; approximation. This function just implements Newton's formula for

; ONE step and does not iterate. It is called from function polyzero.

; Returns a number or NaN if the next value could not be calculated.

(defun newton (polycoeffs xn)

(let* (( yyd (polyeval polycoeffs xn) )

( y   (first yyd) )

( yd  (second yyd) )    ; PS: (nth 1 yyd) = (elt yyd 1) = (second yyd)

)

(- xn (/ y yd) )         ; x[n+1] := x[n] + f(x[n]) / f'(x[n])

)

)

; -------------------------------------------------------------------------------

; (polyreduce polycoeffs x0)

;

; Intepretes the list polycoeffs as coefficients (ordered by growing

; powers) of a polynominal function and returns the polynomial which

; results from dividing polycoeffs by (x-x0)

;

; This is done by using Horner's algorithm:

;   if    f(x) = u[n]*x^n + u[n-1]*x^(n-1) + ... + u[1]*x + u[0]

;   then  g(x) = f(x) / (x-x0)

;   is         = v[n-1]*x^(n-1) + v[n-2]*x^(n-2) + ... + v[1]*x + v[1]

;   where v[k] = v[k+1]*x0 + u[k]

(defun polyreduce (polycoeffs x0)

(reverse (horner 0 (reverse polycoeffs) x0))

)

(defun horner (pv polycoeffs x0)       ; pv = previous v (from previous iteration)

(if (> (length polycoeffs) 1)

(let (( nv  (+ (* pv x0) (first polycoeffs)) )) ; (first polycoeffs) = u[k]

(cons nv

(horner nv (rest polycoeffs) x0)

)

)

)

)

; -------------------------------------------------------------------------------

; (polyeval polycoeffs x)

;

; Intepretes the list polycoeffs as coefficients (ordered by growing

; powers) of a polynominal function which is evaluted for x. The function

; value and the value of the derrived function at point x are returned as list.

;

; Source: This is copied from Ex. 19.

(defun polyeval (polycoeffs x)

(cond ((null polycoeffs)

nil)                         ; return nil for empty lists

((null (rest polycoeffs))

(cons (first polycoeffs) '(0)) )  ; return c and 0 for const poly.

(t

(let* ((evalrest (polyeval (rest polycoeffs) x) )  ; otherwise...

(p_x     (first evalrest)  )

(p_prime (second evalrest) )

)

(cons (+ (* p_x x) (first polycoeffs))

(cons (+ p_x (* p_prime x)) nil)

)

)

)

)

)